Title: Load Securement for Open Deck Loads

Section: Achieving a Static Load and Managing Friction

Module: Module 7

Section Summary:

This section explains how to create a static load that cannot move in any direction and why this is essential for safe transport. It clarifies how applied force and friction work together, describes how to increase static friction using tools such as friction mats, and shows how V-boards help distribute pressure and protect freight during securement.

_______________________________________

Achieving a Static Load and Managing Friction

Learning Objectives

By the end of this section, you will be able to:

- Explain what a static load is and why it is essential for safe transport.

- Describe how applied force and friction interact to keep cargo from moving.

- Identify ways to increase static friction using tools such as friction mats.

- Explain how V-boards contribute to load stability and freight protection.

- Apply top pressure concepts to improve overall load securement.

Lesson Content

1. Understanding the Goal: Creating a Static Load

A static load is one that cannot move in any direction—forward, backward, sideways, or vertically. Your goal is to convert every load into a static load before moving your vehicle. You accomplish this by applying enough securement force and friction to lock the freight into position.

Static loads help prevent shifting, sliding, or falling during acceleration, braking, or turning. When you understand the forces acting on your load, you can take deliberate steps to prevent movement and maintain compliance with federal securement requirements.

2. The Role of Applied Force and Friction

Every load is affected by forces as the truck moves. Acceleration pushes the load backward, braking pushes it forward, and cornering can shift weight side-to-side. Friction is your first line of defense against these forces. Static friction—the resistance that prevents an object from starting to move—is especially important.

To increase static friction, ensure strong contact between the load and the trailer deck. Clean surfaces, proper load placement, and securement devices that apply downward pressure all help create the friction needed to keep cargo from shifting.

Friction mats are an effective way to increase static friction. They provide a high‑friction surface between the load and the deck, making it harder for freight to slide. Using friction mats reduces the amount of force your securement devices must provide and boosts overall load stability.

3. Enhancing Load Stability with V-Boards and Top Pressure

V-boards help distribute strap or chain pressure across a wider area of the freight, preventing edge damage and allowing you to tighten your securement more confidently. By spreading the force evenly, V-boards protect the cargo and support your goal of creating a static load.

Along with friction, top pressure—created by straps, chains, or binders pulling downward—helps lock the load in place. The combination of friction at the base and downward pressure from above reduces the likelihood of movement in any direction.

Effective top pressure keeps the load firmly seated against the trailer deck, increasing static friction and improving stability. Used together, friction mats, V-boards, and proper securement techniques help you achieve a true static load.

Scenario / Case Study

You are hauling a bundle of steel pipe on an open deck trailer. The smooth pipes tend to slide when the truck accelerates or stops abruptly. To reduce this risk, you place friction mats under the bundle to increase grip between the pipe and the trailer deck. To protect edges and distribute strap pressure evenly, you add V-boards before tightening your straps.

During your final walk‑around, you confirm that the top pressure is sufficient and that the load is firmly seated. With friction mats increasing static friction and V-boards preventing strap damage, the bundle now behaves as a single, stable unit.

Reflective Questions:

- How would this load behave differently without friction mats?

- What risks could occur if V-boards were not used on this type of freight?

- How does top pressure contribute to achieving a static load in this scenario?

Knowledge Check

1. What is the primary goal when securing an open deck load?

A) Reducing the number of straps needed

B) Ensuring quick loading and unloading

C) Creating a static load that cannot move

D) Allowing limited movement during transport

2. What does static friction help prevent?

A) Load expansion

B) Load movement from a resting position

C) Strap wear and tear

D) Trailer deck bending

3. How do friction mats improve load securement?

A) By reducing the need for V-boards

B) By allowing straps to be placed loosely

C) By increasing friction between the load and the deck

D) By elevating the load for airflow

4. What is the main purpose of V-boards?

A) To replace friction mats entirely

B) To increase the deck height

C) To distribute strap pressure and protect freight

D) To add weight to the load

5. What does top pressure help accomplish?

A) Reduces strap tension requirements

B) Increases downward force to enhance static friction

C) Pushes the load sideways for balance

D) Eliminates the need for friction entirely

Correct answers:

1) C

2) B

3) C

4) C

5) B