Title: Load Securement for Open Deck Loads

Section: Advanced Static Load Principles and Damage Prevention

Module: Module 9

Section Summary:

This section expands on the principles required to create a stable, static load. It reinforces the importance of using equal but opposite forces with at least four securement points, explains why light cargo is more prone to shifting, and outlines how improper pressure can damage soft or palletized freight. You will also learn how gut/belly straps and friction‑increasing tools help prevent sliding and protect delicate materials.

_______________________________________

Advanced Static Load Principles and Damage Prevention

Learning Objectives

By the end of this section, you will be able to:

• Explain why creating a static load requires equal but opposite securement forces.

• Identify risks associated with securing light cargo.

• Apply techniques such as gut/belly straps and friction brakes to reduce sliding.

• Recognize how improper pressure can damage soft or palletized freight.

• Determine where strong points exist on crates and other delicate cargo.

Lesson Content

1. Creating Equal but Opposite Forces

Creating a static load means preventing movement in every direction. To achieve this, you must apply securement forces that are equal but opposite so the load cannot shift forward, backward, or sideways.

Using at least four securement points is essential for balanced pressure. Each point counteracts another, helping the load resist forces such as braking, accelerating, turning, or wind.

This principle applies to all open deck freight but is especially important when handling lighter cargo that is more easily affected by outside forces.

2. Preventing Movement of Light Cargo

Light cargo presents unique challenges. Because it has less natural friction against the deck, it is more susceptible to sliding, swaying, or shifting from wind or uneven pressure.

Multiple straps placed at balanced angles help remove gaps and reduce movement. Friction‑increasing tools—such as friction mats—can also support stability.

Gut or belly straps are particularly effective for lighter loads. By wrapping around the product and increasing downward force, they improve friction and help the load grip the deck more securely.

3. Avoiding Damage from Direct Pressure

Securement must be tight enough to hold the load, but excessive or poorly placed pressure can damage soft or palletized freight. Loads packed in bags or crates can deform or crush if tension is applied incorrectly.

Always look for the strong points of crates or totes—typically located at the ends—so strap tension is applied where the structure can handle it.

For materials such as sheet rock, insulation, or plastic‑wrapped goods, corner protectors or v‑boards help distribute pressure, protect edges, and reduce the risk of tearing or crumbling.

Scenario / Case Study

You are hauling several pallets of lightweight insulation sheets on an open deck trailer. The freight is wrapped in plastic and stacked in tall bundles. Because the load is light, you initially place a single strap over the center of each bundle. A few miles down the road, you notice in your mirrors that the bundles are tilting slightly.

You pull over to reassess. The single straps are applying downward pressure but not enough opposing force to prevent sideways movement. You add angled straps for balance, v‑boards to protect the soft edges, and belly straps to increase friction and eliminate small gaps.

Reflective questions:

• What role did the weight of the cargo play in the initial shifting?

• How did adding belly straps improve static resistance?

• What damage might have occurred without the v‑boards?

Knowledge Check

1. What is required to create a static load?

A. Using only heavy‑duty straps

B. Equal but opposite forces with multiple securement points

C. Strapping only the top layer of cargo

D. Securing the load with chains only

2. Why is light cargo more likely to shift?

A. It cannot be strapped tightly

B. It has lower friction and is more affected by wind

C. It is too bulky for angled straps

D. It is usually shrink‑wrapped

3. What is the main purpose of gut/belly straps?

A. To raise the load higher on the deck

B. To increase friction and eliminate small gaps

C. To secure only the bottom pallet

D. To replace corner protectors

4. Where should you apply strap pressure on crates or totes?

A. Anywhere along the sides

B. Only on the top surface

C. At the weakest point

D. At the strong points, usually at the ends

5. Why are corner protectors or v‑boards used with soft or fragile materials?

A. To add weight to the load

B. To make strapping unnecessary

C. To distribute pressure and prevent damage

D. To reduce the number of straps needed

Correct answers:

1) B

2) B

3) B

4) D

5) C