## Module 2 Quiz — Escort Operations, Communication, and Traffic Control

20 multiple-choice questions, scenario-based where appropriate

### Escort Roles & Duties

- The primary purpose of escort vehicles is to:
 A) Clear traffic by speeding ahead
 B) Warn the public and assist safe movement of the load
 C) Manage driver timecards
 D) Replace the driver's responsibility

 Correct Answer: B

- A high-pole escort is required when:
 A) The load exceeds legal width
 B) Height clearance must be monitored
 C) Load is especially long
 D) Traveling at night

 Correct Answer: B

- The front escort typically travels:
 A) Directly behind the load
 B) ¼ to ½ mile ahead when safe
 C) On the shoulder
 D) At the same bumper as the truck

 Correct Answer: B

- The rear escort's primary duty is to:
 A) Determine the route
 B) Watch traffic behind and warn the driver
 C) Inspect permits
 D) Operate the high-pole

 Correct Answer: B

- The driver and escort share responsibility, but the final legal responsibility belongs to:
 A) The escort
 B) The permit office
 C) The driver and carrier
 D) The customer

 Correct Answer: C

### Communication Protocols

- Radios must be tested:
 A) Once a month
 B) Once per new escort rotation
 C) Daily before movement
 D) Only during emergencies

 Correct Answer: C

- A sterile radio channel means:
 A) No communication allowed
 B) Only essential safety communication
 C) Music only
 D) Texting instead of radio use

 Correct Answer: B

- If radio communication becomes unclear while approaching an overpass:
 A) Proceed slowly
 B) Stop movement until communication is restored and safe
 C) Ask the rear escort to signal instead
 D) Honk to warn traffic

 Correct Answer: B

- Proper radio communication should be:
 A) Fast and casual
 B) Clear, calm, and professional
 C) Full of slang
 D) Aggressive when traffic is heavy

 Correct Answer: B

- If the primary channel becomes busy with outside chatter, escorts should:
 A) Ignore it
 B) Switch to backup channel as planned
 C) Yell louder
 D) Stop talking completely

 Correct Answer: B

### Traffic Control & Flagging

- A rear escort notices several cars stacking behind the load. The correct action is:
 A) Speed up
 B) Plan a safe pull-off location when available
 C) Stop in the lane immediately
 D) Ignore the cars

 Correct Answer: B

- When flagging, the flagger should:
 A) Stand in the roadway
 B) Stand where vehicles can clearly see them
 C) Face away from traffic
 D) Avoid using flags

 Correct Answer: B

- Hazard lights should be used when vehicle speed is:
 A) Under 30 mph
 B) Above 60 mph
 C) Under 55 mph
 D) Only at night

 Correct Answer: A

- Traffic control is important because:
 A) It is optional
 B) It protects the public and prevents unsafe passing
 C) It speeds up delivery
 D) It replaces escort duties

 Correct Answer: B

- If a driver encounters aggressive public motorists, they should:
 A) Race ahead
 B) Maintain calm communication and safety control
 C) Block traffic intentionally
 D) Engage the motorist

 Correct Answer: B

### Scenario-Based Questions

- A rear escort sees a vehicle trying to pass on a blind curve. They should:
 A) Speed ahead to chase them
 B) Warn the driver and help prevent unsafe passing
 C) Ignore it
 D) Signal the motorist to pass faster

 Correct Answer: B

- Approaching a narrow bridge, the escort calls for a stop. The driver should:
 A) Ignore the call and continue
 B) Stop safely and wait for the all-clear
 C) Speed across
 D) Take a detour without notice

 Correct Answer: B

- A high-pole touches a cable lightly. The correct action is:
 A) Continue cautiously
 B) Stop immediately and verify clearance
 C) Ask another pilot car
 D) Call dispatch later

 Correct Answer: B

- Escort says “sterile channel” while approaching construction. This means:
 A) No one may speak
 B) Only safety-critical communication allowed
 C) Switch off radios
 D) Use hand signals only

 Correct Answer: B

- During a route, radio interference makes communication unclear. The team should:
 A) Continue because time is tight
 B) Stop in a safe location and establish clear communication
 C) Turn radios off
 D) Signal by horn

 Correct Answer: B

### End of Module 2 Quiz