## Module 2 — Section 2.1

### Escort & Pilot Car Roles and Responsibilities

### Learning Objectives

By the end of this section, the learner will be able to:

- Explain the purpose of escort and pilot vehicles in oversize-load transport

- Identify duties of front escorts, rear escorts, and high-pole vehicles

- Understand how escorts communicate hazards and support safe movement

- Recognize the importance of coordination between drivers and escort teams

### Lesson Content

Oversize loads require coordinated movement to ensure safety. Escort and pilot vehicles are essential partners in that process. Their primary purpose is:

To protect the public, assist the driver, and ensure safe passage of the load.

Escort vehicles serve as the driver’s extended visibility, offering early warnings and traffic control support during movement.

### 1. Purpose of Escort and Pilot Vehicles

Escort and pilot vehicles:

- Warn the public of an oversize load

- Assist with safe positioning and spacing

- Monitor roadway conditions and traffic behavior

- Identify clearances, hazards, and restricted areas

- Support safe lane changes, turns, and stops

- Help the operator maintain permit compliance

Their support helps reduce accidents, delays, and property damage.

### 2. Front Escort Duties

Front escorts lead the movement and provide advanced warning.

Responsibilities include:

- Scanning ahead for hazards, road conditions, and obstacles

- Reporting low bridges, construction, tight turns, debris, and sudden traffic slowdowns

- Guiding the truck through intersections and tight maneuvers

- Advising when it is safe to merge or exit highways

A front escort typically travels one-quarter to one-half mile ahead when conditions allow.

### 3. Rear Escort Duties

Rear escorts follow the load and monitor traffic behind the movement.

Responsibilities include:

- Watching for traffic buildup

- Warning the driver about aggressive or unsafe drivers

- Assisting with lane changes and turns

- Calling out equipment issues, smoke, debris, or tire problems

- Helping find safe pull-off points when vehicles stack behind

When three or more vehicles (or up to five in some states) accumulate, a safe pull-off plan should be executed when possible.

### 4. High-Pole Escort Responsibilities

High-pole escorts are used for taller loads requiring clearance checks.

Responsibilities include:

- Measuring overhead clearance for bridges, signs, utility wires, and structures

- Reporting any contact, near-contact, or brush events

- Stopping movement if height clearance is uncertain

- Providing early warning to prevent sudden stops and load damage

If the pole touches anything, the load does not proceed until clearance is verified and safe.

### 5. Teamwork and Communication

Drivers and escort crews function as a coordinated safety team. Success relies on:

- Clear pre-trip planning and route review

- Selecting a primary and backup radio channel

- Constant and professional radio updates

- Immediate communication of route deviations or hazards

- Agreed-upon emergency procedures and sterile channel use

Escort vehicles do not simply follow the load. They actively support safety.

### Key Takeaway

Escorts enhance safety and assist with movement, but the driver remains ultimately responsible for the load and regulatory compliance.

### Scenario / Case Study

Scenario:
 You are transporting a wide load on a rural two-lane road. The rear escort notifies you that several vehicles are building up behind you and drivers appear impatient.

Questions:

- What action should be taken?

- Why is communication important here?

- What if no safe pull-off is immediately available?

Correct action:
 Maintain communication with the escort. Plan a safe pull-off location; do not stop in unsafe locations. Slow gradually and reenter traffic only when safe.

This ensures public safety and prevents risky passing behavior.

### Knowledge Check

- The primary purpose of escort vehicles is to:
 A) Block traffic
 B) Assist the driver and warn the public
 C) Navigate only for the driver
 D) Speed ahead and clear lanes

 Correct answer: B

- Front escorts primarily:
 A) Travel behind the load
 B) Monitor and warn of hazards ahead
 C) Handle driver scheduling
 D) Perform on-site loading

 Correct answer: B

- Rear escorts primarily:
 A) Measure height clearances
 B) Warn the driver about traffic buildup
 C) Determine legal routes
 D) Lead the convoy

 Correct answer: B

- High-pole escorts are required when:
 A) Traveling in heavy rain
 B) Load height requires clearance verification
 C) There is no rear escort available
 D) Load width exceeds 12 feet

 Correct answer: B

- If traffic stacks behind the load:
 A) Speed up to reduce frustration
 B) Plan a safe pull-off when feasible
 C) Ignore it
 D) Stop immediately regardless of location

 Correct answer: B

### Summary

Escort vehicles are critical safety partners in oversize-load transport. They identify hazards, support navigation, watch traffic, and communicate essential information to the driver. Safe movement depends on teamwork, discipline, communication, and awareness.