## Module 2 — Section 2.2

### Communication Protocols & COMS Procedures

### Learning Objectives

By the end of this section, the learner will be able to:

- Explain why communication is critical in oversize-load operations

- Identify proper radio setup and use procedures

- Understand sterile channel policy and when to apply it

- Recognize expectations for clear, professional communication with escort teams

### Lesson Content

Proper communication is one of the most important factors in safely moving an oversize load. Reliable, clear, and professional communication prevents errors, route deviations, hazards, and accidents.

In oversize hauling, communication is not casual — it is planned, controlled, and continuous.

### 1. Importance of Communication

Oversize operations require coordination between:

- Truck driver

- Front escort vehicle

- Rear escort vehicle

- High-pole (if applicable)

- Additional support vehicles or law enforcement (where applicable)

Effective communication ensures:

- Route safety

- Hazard awareness

- Traffic coordination

- Accurate spacing and timing

- Emergency readiness

Poor communication is a leading cause of preventable oversize transportation incidents.

### 2. Radio Setup and Daily COMS Check

Before movement begins, all radios must be tested and confirmed operational.

Required steps:

- Test CB radios at the start of each day

- Verify primary and backup channels

- Confirm microphone clarity and volume

- Ensure everyone hears and transmits clearly

- Review emergency channel use

- Confirm common terminology

The driver should not move the load until communication is verified.

If you cannot talk, you cannot move.

### 3. Radio Channel Selection

Communication protocols include:

- Primary channel — for standard movement communication

- Backup channel — used if interference occurs

- Sterile channel — reserved for safety-only communication

Escort vehicles and drivers agree on channels during pre-trip planning.

Citizen chatter or non-movement talk should not interfere with operations.

### 4. Sterile Radio Policy

A sterile channel means radio use is restricted to essential communication only.

Used when:

- Navigating tight or complex areas

- Approaching bridges or railroad crossings

- Confirming clearances or route conditions

- Handling emergency or unexpected situations

- Traffic conditions require heightened focus

Any team member may call for a sterile channel when safety requires it.

Sterile channel = No chatter. Only safety communication.

### 5. Communication Conduct and Clarity

Professional oversize communication requires:

- Clear, calm, and respectful speech

- Speaking slowly and clearly

- Short, direct messages

- Avoiding slang, mumbling, or emotional language

- Using standard callouts for hazards, turns, clearances, and stops

Escort and driver communication must remain professional at all times.

A professional callout example:

“Traffic slowing ahead, prepare to reduce speed,”
 not
 “Hey watch out up here.”

### 6. Route Deviations and Hazard Calls

If an escort or driver identifies a deviation or hazard, they must call it out immediately.

Examples:

- Driver drifting off route

- Low wires or tree limbs

- Unexpected detours or construction

- Emergency stopping needs

- Aggressive or unsafe public drivers

If uncertain — stop safely and verify.

When in doubt, stop and communicate.

### Scenario / Case Study

Scenario:
 You are moving a tall load and the high-pole escort radio crackles with interference. Communication becomes unclear as you approach an overpass.

Questions:

- What should the driver do?

- Should the load continue based on unclear communication?

Correct action:
 Stop movement immediately in a safe area.
 Verify radio function and clearance.
 Do not proceed until communication is clear and confirmed.

Oversize hauling demands certainty — not assumptions.

### Knowledge Check

- Why is communication critical in oversize transport?
 A) It simplifies paperwork
 B) It ensures coordinated and safe movement
 C) It impresses the customer
 D) It prevents fuel loss

 Correct answer: B

- Radios must be tested:
 A) Once a week
 B) Only before escort changes
 C) Daily before movement
 D) Only when troubleshooting

 Correct answer: C

- A sterile channel is used when:
 A) Drivers want to chat casually
 B) Passing a hazard or needing focused communication
 C) Radios are off-frequency
 D) Escort vehicles are not aligned

 Correct answer: B

- The correct response to unclear radio communication about hazards is:
 A) Continue slowly
 B) Stop and verify communication before moving
 C) Rely on a GPS app
 D) Ask the escort to text instructions

 Correct answer: B

- Professional radio etiquette includes:
 A) Fast talking and slang
 B) Interrupting others
 C) Clear, calm language and standard callouts
 D) Speaking only when spoken to

 Correct answer: C

### Summary

Communication is the backbone of safe oversize-load transport. It must be:

- Constant

- Clear

- Professional

- Verified before movement starts

- Adapted to conditions (sterile channel when needed)

When coordination fails, risk increases. When communication is strong, the move is safe and efficient.