## Module 2 — Section 2.3

### Traffic Control & Flagging Responsibilities

### Learning Objectives

By the end of this section, the learner will be able to:

- Explain the purpose of traffic control in oversize-load movement

- Identify proper flagging procedures and positioning

- Understand when escort vehicles must assist with traffic flow

- Recognize safety and courtesy responsibilities to the public

### Lesson Content

Oversize loads move slower, take more space, and require additional time and room for turns, lane changes, and stopping. Because of this, traffic control and flagging are essential to keep the public, driver, and crew safe.

Professional traffic control protects:

- The traveling public

- The driver and escort team

- The load and equipment

- Roadway property and infrastructure

Escort drivers and flaggers are safety partners, not spectators. Their role is active and anticipatory.

### 1. Purpose of Traffic Control

Traffic control during oversize movement ensures:

- Safe passage of the load through tight or complex areas

- Reduced risk of collisions or unsafe passing

- Smooth flow of public traffic when possible

- Proper management of congestion situations

Good traffic control prevents accidents caused by impatience, confusion, or blind spots around the load.

Public drivers often do not understand oversize operations — it is the escort team’s responsibility to manage the situation professionally.

### 2. Rear Escort Traffic Monitoring

Rear escorts must continuously monitor traffic behind the load.

Responsibilities include:

- Watching for vehicles stacking behind the convoy

- Alerting the driver of aggressive or unsafe behavior

- Preventing dangerous passing attempts when possible

- Coordinating safe pull-offs

Standard guideline:
 When three or more vehicles are stacking behind (or five, depending on state law), a safe pull-off should be planned as soon as a safe area is available.

Stopping must never be done in an unsafe area.

### 3. Flagging Procedures

Flagging is used to:

- Direct traffic safely around the load

- Stop traffic when necessary

- Signal turns, lane shifts, and backing maneuvers

- Provide visual cues to drivers approaching blind spots

Basic flagging rules:

- Stand where oncoming vehicles can see you clearly

- Never stand in the lane of moving traffic

- Maintain a safe position on the shoulder or protected area

- Face oncoming traffic and remain alert

- Use a flag or paddle to clearly signal to motorists

- Be visible, direct, and predictable

Courtesy counts. Professionalism builds respect and cooperation.

### 4. Texas and State-Specific Requirements

Example requirement:
 Texas law requires pilot cars to be flagged on all four corners.

State laws vary regarding:

- Flag size

- Vest and PPE requirements

- Light and banner rules

- Number of pilot cars required

- Pull-off thresholds for stacked traffic

Drivers and escorts must know the laws in all states traveled. Permit instructions are legally binding.

### 5. Hazard Lights and Movement Speed

Driver expectations:

- Hazard lights should be used when traveling under 30 mph

- Speed should match safe operational limits, not traffic pressure

- Sudden lane changes should be avoided unless required for safety

Rear escorts assist by:

- Warning of merging or passing vehicles

- Creating safe space for the convoy when necessary

- Communicating lane change timing to the driver

The priority is always safety, not convenience.

### 6. Coordination and Courtesy

Professional drivers and escort teams:

- Show courtesy to the public

- Avoid unnecessary delays

- Maintain calm, respectful communication

- Manage traffic flow with caution and awareness

A courteous escort team avoids escalation and promotes a safe travel environment.

### Scenario / Case Study

Scenario:
 You are approaching a narrow bridge on a two-lane road. Traffic is building behind the load. The rear escort advises that an impatient driver is attempting to pass on a blind hill.

Questions:

- What should the escort do?

- How should the driver respond?

- What safety steps should be taken before crossing the bridge?

Correct action:

- Escort blocks unsafe passing if necessary

- Driver slows and prepares to stop if hazard develops

- Traffic cleared before entering bridge section

- Safe communication and control maintained

Uncontrolled passing around oversize loads is dangerous. Prevention is the goal.

### Knowledge Check

- The purpose of traffic control is to:
 A) Move traffic faster
 B) Safely manage movement around an oversize load
 C) Allow escort vehicles to drive aggressively
 D) Reduce escort responsibilities

 Correct answer: B

- When three or more vehicles stack behind the convoy, the team should:
 A) Stop immediately in the road
 B) Speed up
 C) Locate a safe pull-off location when possible
 D) Ignore it

 Correct answer: C

- Flaggers must:
 A) Stand in the lane to get attention
 B) Stand where traffic can see them safely and clearly
 C) Avoid making hand signals
 D) Only flag at night

 Correct answer: B

- Hazard lights should be used when:
 A) Traveling under 30 mph
 B) Passing vehicles
 C) Off duty
 D) Only at night

 Correct answer: A

- Courtesy and communication matter because:
 A) They create a positive safety culture
 B) They eliminate the need for permits
 C) They reduce fuel cost
 D) They are optional

 Correct answer: A

### Summary

Traffic control and flagging are fundamental to safe oversize-load transportation. Escort teams and drivers must work together to:

- Manage traffic safely

- Prevent risky passing behavior

- Communicate clearly and consistently

- Follow state laws and permit instructions

- Maintain courtesy and professionalism

Safety and public protection are always the priority.