## Module 3 — Section 3.2

### Route Surveys & Hazard Identification

### Learning Objectives

By the end of this section, the learner will be able to:

- Explain what a route survey is and why it is required

- Identify common hazards during oversize-load transport

- Understand time-sensitivity of route survey validity

- Take proper action when encountering unexpected route issues

### Lesson Content

Route surveys are a core requirement in moving oversize and over-dimensional freight. They are the foundation of safety planning. A route survey confirms that the planned path is physically safe, legally allowed, and properly prepared for the movement of the load.

A route survey is not optional—it is a required, documented, pre-movement safety check created to prevent collisions, infrastructure damage, road shutdowns, and dangerous emergency stops.

### Route Survey Purpose

A route survey provides the carrier and driver with verified route information, including:

- Overhead clearances (bridges, wires, signs, lights, trees)

- Ground clearance hazards (railroad tracks, dips, uneven pavement, slopes)

- Road width and lane availability

- Turning radius evaluations

- Bridge weight limits and roadway strength

- Required advance notices (especially railroads or utilities)

The route survey confirms that the planned route can physically handle the load being moved.

### Critical Route Survey Rules

- Route surveys should not be older than 30 days.
 Conditions change—construction, road maintenance, utilities, and weather can all alter clearance and access.

- When in doubt, do not proceed.
 Stop, communicate, verify. Hazards must be solved before moving forward.

- Route surveys must align with the permit.
 The permitted route is legally binding. Deviations may result in fines, shutdowns, or damage liability.

- Escort vehicles and the driver must review the route together.
 This includes planned stops, hazards, narrow sections, rail crossings, and bridges.

- Pre-trip discussions should include emergency plans and sterile-channel radio procedures when approaching hazard zones.

### Recognizing Hazards on the Road

Hazards may include:

- Low bridges, overpasses, or wires

- Railroad crossings and uneven track beds

- Road crown issues affecting load balance

- Curves with limited turning radius

- Narrow lanes, shoulders, or bridges

- Steep grades affecting speed control

- Construction zones or temporary lane shifts

- Overhanging tree limbs or structures

A professional driver and escort team remain vigilant. Rail crossings deserve special attention — the carrier should confirm the railroad was notified, especially for slow or low-clearance equipment.

### Driver & Escort Action Steps When Hazards Occur

If a hazard is observed or called out:

- Stop safely and immediately

- Confirm hazard severity

- Switch to sterile radio if not already active

- Review permit and route survey

- Adjust or request alternate routing if needed

- Never gamble on clearance

- Document issues if route changes are required

A safe decision is never the wrong decision.

### Scenario / Case Study

Scenario:
 You are approaching a railroad crossing that appears steeper than expected. The escort calls out that the high-pole bounced at the previous crossing. The permit notes the crossing but the route survey is 45 days old.

Questions:

- Should you proceed?

- What steps must be taken?

- Who must be notified?

Correct action:
 Stop and reassess. Notify dispatch. Contact railroad if needed. Confirm safe crossing height and clearance. The load does not move until safety is verified.

Route surveys are only as good as their accuracy and age — conditions change.

### Knowledge Check

- A route survey is considered valid for:
 A) 6 months
 B) 90 days
 C) No more than 30 days
 D) Until weather changes

 Correct Answer: C

- If a route condition appears different from the survey:
 A) Continue through slowly
 B) Stop and verify before moving
 C) Ask another trucker
 D) Call the shipper later

 Correct Answer: B

- Railroad crossings require:
 A) Fast crossing to avoid blocking trains
 B) Prior notification to the railroad when necessary
 C) No special planning
 D) Removing escorts to avoid delays

 Correct Answer: B

- If the escort reports the high-pole made contact:
 A) Proceed carefully
 B) Stop immediately and assess
 C) Increase speed
 D) Ignore unless damaged

 Correct Answer: B

- The route survey must align with:
 A) Driver preference
 B) Weather conditions
 C) The permit
 D) Road signs only

 Correct Answer: C

### Summary

Route surveys are the foundation of oversize-load route safety. They confirm physical clearances, identify hazards, and ensure legal compliance. Drivers and escort teams must review surveys together, monitor real-time hazards, and stop movement when conditions change.

A route survey is a living safety document — and when it is outdated or inaccurate, the movement pauses.