## Module 4 — Section 4.1

### Vehicle & Equipment Inspection Requirements

### Learning Objectives

By the end of this section, the learner will be able to:

- Explain the purpose of pre-trip and post-trip inspections for oversize hauling

- Identify the critical components of a heavy haul / OSOW inspection

- Understand why additional checks are required beyond standard DOT inspections

- Recognize when equipment must be repaired or movement stopped

### Lesson Content

Oversize and heavy haul equipment experiences higher stress, increased load forces, and tighter operational tolerances than standard trucking. Because of this, inspection standards must be higher than minimum DOT requirements.

A missed detail on an oversize load can lead to:

- Load shift or securement failure

- Bridge or roadway damage

- Tire blowouts under extreme weight

- Hydraulic, electrical, or steering issues

- Traffic hazards and vehicle accidents

- Permit violations and shutdowns

Professional drivers perform thorough inspections before, during, and after every movement.

### 1. Pre-Trip Inspection Expectations

A proper pre-trip inspection for oversize work includes:

- Full DOT inspection (lights, brakes, fluids, steering, tires, suspension, etc.)

- Verification of tire condition and inflation on steer, drive, and trailer axles

- Fifth wheel and kingpin inspection

- Electrical and air line connections

- Trailer load-bearing components (jeeps, dollies, boosters)

- Hitch, shackles, chains, straps, binders, and winch lines

- Safety equipment (flags, banners, lights, reflectors)

- Power unit load rating and condition

Oversize loads often involve multi-axle trailers, steerable dollies, spreaders, or jeeps, all of which require additional attention.

If anything is questionable — repair first, move second.

### 2. Load Securement & Structural Checks

Oversize freight securement must:

- Meet or exceed FMCSA securement rules

- Match manufacturer recommendations

- Be inspected regularly during transit

- Remain tight and undamaged

Securement checks include:

- Chain and strap condition (no cuts, cracks, or kinks)

- Binder tightness

- Winch line seating and condition

- Corner protectors and softeners when needed

- Anchor points and trailer structural integrity

Remember:
 More load weight = more securement responsibility.

### 3. Escort & Safety Equipment Verification

Before movement begins, confirm:

- Oversize banners installed and visible

- Red or fluorescent flags on all four corners

- Amber rotating or strobe lights functional

- Radios operational and tested

- Spare securement equipment available

- Escort requirements match permit

If escort equipment is missing or their vehicle is not inspection-ready, the load does not move.

### 4. En Route Inspections

Oversize loads require checks at:

- First 50 miles

- Every 150 miles thereafter

- Every change in escort vehicle

- Before entering challenging turns, mountain passes, or restricted zones

During transit, check:

- Tire temperature & pressure

- Chain and strap tension

- Electrical/air lines clearance & tension points

- Jeep/dolly connection points

- Brake performance and travel

If weather, roadway vibration, or heavy braking occurs — increase frequency of checks.

### 5. Post-Trip Inspection

After movement:

- Inspect all equipment and securement hardware

- Document any wear, damage, or failures

- Report issues to maintenance or dispatch

- Plan repairs before the next move

Professional operators never assume everything will "still be good tomorrow." Heavy loads cause real fatigue and wear.

### Scenario / Case Study

Scenario:
 During a pre-trip walk-around, you notice a binder has visible wear and a chain is showing stretch marks from previous heavy tension. The load is scheduled to move in 30 minutes and the customer is waiting.

Questions:

- Do you move the load?

- Why or why not?

- What action should you take?

Correct response:

Do not move. Replace the damaged securement immediately.
 Customer deadlines do not override safety or compliance.

Equipment failure under load is dangerous and preventable.

### Knowledge Check

- Oversize load inspections must:
 A) Meet or exceed standard DOT requirements
 B) Match normal pallet freight procedures
 C) Be done once per week
 D) Be performed only by mechanics

 Correct Answer: A

- Securement on an oversize load must be checked:
 A) Once a day
 B) At the first 50 miles and regularly after
 C) Only if a strap breaks
 D) Only during DOT inspections

 Correct Answer: B

- When an escort vehicle is missing proper flags or lighting:
 A) Continue if weather is clear
 B) Move slowly and avoid major roads
 C) Do not move until corrected
 D) Follow behind another oversize load

 Correct Answer: C

- A chain with visible stretch should:
 A) Be tightened
 B) Be monitored only
 C) Be replaced before movement
 D) Be used only for lighter loads

 Correct Answer: C

- Proper oversize inspection frequency is required because:
 A) Customers demand it
 B) Oversize equipment experiences greater stress and vibration
 C) It speeds up delivery
 D) It reduces fuel burn

 Correct Answer: B

### Summary

Oversize transportation places high stress on equipment, securement, and support systems. The driver is the final gatekeeper of safety. Thorough inspection, frequent checks, and immediate correction of issues protect:

- The driver

- The public

- The equipment

- The customer’s load

- Company safety record

Professional oversize drivers never gamble with equipment condition.