## Module 4 — Section 4.2

### Trailer Types & Load Configuration Basics

### Learning Objectives

By the end of this section, the learner will be able to:

- Identify common trailer types used in oversize and heavy-haul transport

- Understand basic load placement and configuration principles

- Recognize how trailer type affects routing and permit requirements

- Explain why proper trailer selection and configuration matters for safety and compliance

### Lesson Content

Oversize and heavy-haul operations rely on specialized trailers engineered to support unique freight dimensions and weight. Choosing the right trailer — and configuring the load correctly — directly impacts:

- Safety

- Stability

- Maneuverability

- Legal compliance

- Bridge and road stress

- Route and clearance requirements

A successful move begins with matching the right trailer to the right load and placing the freight correctly to distribute weight.

### 1. Common Trailer Types Used in Oversize Hauling

Step-Deck (Drop Deck)

- Lower deck height for taller loads

- Common for machinery and agricultural equipment

Double-Drop / Lowboy

- Very low deck to clear height restrictions

- Often detachable neck (RGN) for self-loading equipment

- Ideal for tall and heavy loads like excavators, dozers

Extendable Trailers

- Can stretch for long loads (pipe, girders, bridge beam sections)

- Prevents overhang issues

Multi-Axle Lowboy / Heavy-Haul Combination

- Jeeps, dollies, boosters used for extreme weights

- Increased axle count spreads load over roadway

- Required for high-tonnage freight to meet bridge laws

Steerable Dollies/Boosters

- Rear steer assist for tight turns

- Used when weight demand exceeds standard axle group capacity

Trailer choice dictates:

- Turning radius

- Height capability

- Load concentration points

- Route options (bridge & road rating)

- Escort and permit requirements

### 2. Basic Load Configuration Principles

Weight Distribution Matters
 Proper load placement prevents:

- Overloaded axles

- Poor steering response

- Brake imbalance

- Bridge law violations

Key principles:

- Keep weight balanced between axles

- Avoid excessive rear or forward loading

- Maintain proper fifth-wheel weight

- Respect trailer load rating and concentration limits

Center of Gravity Considerations

High center-of-gravity loads require:

- Slower speeds

- Wider turns

- Additional securement

- Increased caution entering/exiting ramps

Preventing rollover risk is a top priority.

### 3. Why Trailer Choice Impacts Routing

Different trailers have different capabilities and limitations:

- Lowboys require clearance planning (dips, RR crossings)

- Long extendables require wide turns and intersection planning

- Multi-axle trailers may be restricted on smaller bridges

- Steerable axles often require spotter communication

The permit and route survey must match the exact trailer configuration.

A short change in equipment can invalidate a permit.

### 4. Adjustments & Configurations

Drivers must understand optional trailer configurations, such as:

- Adding a jeep or booster for weight distribution

- Using drop axles to reduce ground pressure

- Stretching trailers for over-length freight

- Steering dolly control adjustments

Each configuration comes with training and operational procedures.

If unsure — verify setup with the shop or experienced operator.

### Scenario / Case Study

Scenario:
 You are assigned to haul a tall excavator. The dispatcher suggests using a step-deck to save time, but measurements show the height will exceed legal clearance limits on several structures.

Questions:

- What trailer should be used?

- Why is it important to select correctly?

- What action should you take if pressured to use the wrong setup?

Correct Response:
 Use a lowboy/RGN for height clearance and safe loading.
 Trailer selection is a safety and compliance decision, not a convenience choice.
 Stop and escalate if pressured into unsafe configuration — safety overrides schedule.

### Knowledge Check

- The purpose of multi-axle trailer combinations is to:
 A) Reduce fuel cost
 B) Spread heavy weight safely across axles
 C) Increase truck length for visibility
 D) Reduce driver workload

 Correct Answer: B

- Extendable trailers are used primarily for:
 A) High loads
 B) Long loads
 C) Hazardous loads only
 D) Lightweight machinery

 Correct Answer: B

- Lowboy trailers are chosen because they:
 A) Are fastest to hook up
 B) Reduce driver fatigue
 C) Allow taller loads to clear bridges and obstacles
 D) Carry more fuel

 Correct Answer: C

- Improper weight distribution can result in:
 A) Smooth ride
 B) Steering and braking issues
 C) More fuel efficiency
 D) Quieter operation

 Correct Answer: B

- If assigned a trailer that does not safely meet load height restrictions:
 A) Go slower and hope traffic cooperates
 B) Proceed only at night
 C) Refuse movement until proper equipment is assigned
 D) Ask an escort to watch for bridges

 Correct Answer: C

### Summary

Different loads require different trailer designs and configurations. Proper trailer choice affects:

- Clearance

- Stability

- Axle weight distribution

- Route selection

- Permit validity

The professional oversize operator understands equipment capabilities and never compromises configuration for convenience.

Trailer selection is a safety choice, not a preference.