## Module 4 — Section 4.3

### Safety Equipment, Flags, Signs & Lighting Requirements

### Learning Objectives

By the end of this section, the learner will be able to:

- Identify required safety equipment for oversize-load vehicles

- Understand proper placement of flags, signs, and lighting

- Explain why visibility equipment is essential to public safety

- Ensure compliance with state and federal oversize marking standards

### Lesson Content

Oversize and overweight loads must be clearly marked so the public, law enforcement, and other drivers can easily recognize the size and movement of the vehicle.

Visibility equipment — including banners, flags, lights, and reflectors — reduces collisions, assists escorts, and supports legal compliance.

When oversize markings are missing, blocked, or installed incorrectly, the risk of an incident increases significantly.

These requirements are not cosmetic — they are safety-critical and legally enforceable.

### 1. Oversize Load Signage

“Oversize Load” signs must be:

- Yellow with black lettering

- Highly visible and legible

- Mounted securely at the front of the power unit and the rear of the trailer/load

- Removed or covered when not hauling oversize freight

Signs must not be faded, torn, or obstructed.

Some states allow “Wide Load” or “Long Load” based on the dimension being exceeded — always match state permit language.

If your signs are worn or faded, replace them before movement.

### 2. Warning Flags

Flags help motorists anticipate width and length. They must:

- Be red or fluorescent orange

- Minimum 18” x 18” (larger in some states)

- Mounted at all four corners of the load

- Mark any projections extending past sides or rear

Flags must be clean, bright, and securely fastened.

If wind tears or frays a flag, replace it at the next safe stop.

### 3. Lighting Requirements

Amber lights improve visibility during:

- Low light conditions

- Poor visibility (rain, fog, dust)

- Heavy traffic or high-risk movement zones

Lighting requirements typically include:

- Amber beacon or strobes visible 360 degrees

- Additional amber lighting on escorts, when required

- Flashers used during slow-speed operation (often under 30 mph)

- Marker lights functional and visible on trailer corners

When in doubt, more visibility is better — but follow your state’s lighting rules to avoid overuse citations.

### 4. Escort Vehicle Marking Requirements

Escort vehicles must also meet standards including:

- Oversize signs (front/rear)

- Amber lights visible from all directions

- Safety flags mounted at vehicle corners

- Two-way radios functioning

- High-pole installed when required for clearance

Escort equipment failures mean the load does not move until resolved.

### 5. Equipment Checklist Before Movement

Before beginning transport, verify:

- Oversize signs secured, readable, and correct wording

- Flags properly mounted, no damage or fading

- Lights working and visible

- Radios functional and tested

- Reflective devices and emergency triangles on board

- Spare flags and bungees available

Preparedness prevents delays and roadside violations.

### 6. Public Safety and Legal Compliance

Oversize signage and lighting exists to:

- Protect passing drivers

- Alert approaching traffic

- Help emergency responders identify the vehicle class

- Support safe convoy and escort spacing

- Reduce rear-end and side-swipe collisions

A marked load is a safe load.

Failure to properly mark a load can result in:

- DOT citation

- Permit violation

- Escort or movement shutdown

- Liability in a collision even if not at fault

If visibility is compromised — stop and correct it.

### Scenario / Case Study

Scenario:
 You are midway through your route when you notice your rear flag has torn loose and is dragging. Traffic is moderate, and no safe shoulder is immediately available.

Questions:

- What do you do?

- How should escorts assist?

- Why is this important?

Correct Response:
 Continue to the next safe pull-off point. Notify escorts. Replace the flag before continuing. Safety equipment must be maintained in working condition throughout the trip.

Weak visibility equipment compromises public awareness.

### Knowledge Check

- Oversize signs must be:
 A) Any color or style
 B) Yellow with black lettering
 C) Blue with white lettering
 D) Handwritten

 Correct Answer: B

- Rear flags must be:
 A) Only used on trailers carrying equipment
 B) Red or fluorescent orange and placed on all projecting corners
 C) Used only at night
 D) Optional if lights are used

 Correct Answer: B

- Amber warning lights are used when:
 A) Traveling under 30 mph or in low visibility
 B) Off duty
 C) Parking
 D) At weigh stations only

 Correct Answer: A

- If oversize markings are damaged or missing:
 A) Continue to avoid delay
 B) Stop safely and repair/replace
 C) Remove all other markings
 D) Notify police and continue

 Correct Answer: B

- Escort vehicles must:
 A) Match the color of the load
 B) Have signage, flags, and amber lights
 C) Only use high-poles at night
 D) Use hazard lights only

 Correct Answer: B

### Summary

Oversize safety equipment makes your movement visible, predictable, and legally compliant.

Key rules:

- Signs must be clear, correct, and removed when not in use

- Flags must be intact and placed on all projecting corners

- Amber and marker lights must function and be used properly

- Escorts must meet equipment standards before movement

Professional oversize drivers treat visibility equipment as life-saving tools, not decorations.