## Module 5 — Section 5.3

### Post-Trip Procedures & Reporting

Post-trip procedures are a required part of oversize operations, not an optional formality. The end of travel does not mark the end of responsibility. A controlled shutdown, inspection, and documentation process ensures the equipment remains safe, lessons are captured, and the next move starts from a position of operational readiness.

Oversize hauling creates increased stress on equipment, securement systems, and escort coordination. Neglecting post-trip tasks leads to cumulative wear, preventable failures, and a breakdown of professional standards.

### Key Objectives of Post-Trip Protocol

The post-trip stage is used to:

- Verify the safe release of securement and positioning equipment

- Inspect all components affected by the move, including dollies, jeeps, boosters, and axle groups

- Record route conditions and any operational concerns

- Communicate findings to dispatch, maintenance, or safety personnel

- Prepare equipment and documentation for future permits and route planning

- Capture operational lessons while fresh

Post-trip discipline ensures continuous improvement.

### Controlled Securement Release

Unload and release securement only after the area is verified safe and clear.

Steps include:

- Reconfirm load stability before touching securement devices

- Release chains, binders, and straps gradually and in sequence

- Stand clear of pinch points and tension release paths

- Use wheel chocks, dunnage, and blocking as required

- Never rush equipment unhooking due to yard or customer pressure

- Keep bystanders out of hazard zones

Load release carries risk equal to securement. Precision prevents accidents.

### Equipment Condition Verification

Once the load is clear:

- Inspect all tie-down hardware

  - Chains, binders, ratchets, straps, hooks, winch lines

  - Remove damaged, stretched, or frayed items from service immediately

- Check trailer systems

  - Air lines, hydraulic lines, electric harnesses

  - Suspension links, booster connections, steer dolly mechanisms

  - Brakes, hubs, tires, wheels, torque arms, pins, kingpins

- Verify accessories and high-stress components

  - Oversize banners, mounting points, flag poles, strobe lights

  - Dolly steering bars, lock pins, spreader hardware

- Identify fatigue or stress indicators

  - Leaks, metal stress marks, loose bolts, damaged couplers

  - Tire feathering or cupping caused by cornering forces

If questionable — it is not road-ready.

### Escort & Team Debrief

Hold a short, structured review with escort personnel:

- Confirm route performance and any delays or safety concerns

- Discuss motorist behavior patterns and hazard areas

- Review communication effectiveness

  - Were hazard calls timely?

  - Did sterile-channel engagement work as intended?

  - Were spacing and lane discipline consistent?

- Identify opportunities for improvement

- Reinforce positive performance and professional conduct

A brief professional review reinforces team culture and prevents repeat errors.

### Documentation & Reporting Requirements

Complete documentation while details are fresh:

- Record mileage, time, and stop locations

- Log escort names, contact, and operational notes

- Note any unexpected hazards or detours

- Attach photos of damaged securement or equipment concerns

- Document near-misses, public safety concerns, or road conflicts

- Confirm permit compliance for records

Documentation protects compliance and supports incident defense.

### Maintenance Communication

Immediately report:

- Mechanical issues

- Securement wear or failure indicators

- Tire or axle stress signs

- Light or banner damage

- Dolly, jeep, or booster alignment issues

- Brake heat imbalance or air leak concerns

Provide specifics:

- What occurred

- Where it occurred

- Severity

- Recommended corrective action if known

Accurate reporting reduces downtime and liabilities.

### Post-Trip Key Points

- Unloading and equipment release must be controlled

- Equipment must be inspected immediately after movement

- Records and route notes are completed before leaving

- Findings are communicated, not ignored

- Every trip ends with readiness for the next one

Oversize professionals finish the job all the way.

### Scenario Example

Upon completing delivery of a modular building, you notice chain stretch marks and a faint fluid line on a jeep axle shock. Yard personnel request you clear space quickly to make room for incoming trucks.

Correct Response:

- Do not rush equipment release

- Complete securement removal safely

- Tag the stretched chain out of service

- Report the shock concern to maintenance with photos

- Submit route notes and escort performance review

- Depart only when documentation is complete

If you would refuse to move equipment in poor condition, you must not leave it in poor condition for someone else to move.

### Knowledge Check

- Post-trip duties must be completed:
 A. Only if the load was difficult
 B. Immediately after unloading
 C. Only at the driver's discretion
 D. At the end of the week
 Answer: B

- Securement release should be:
 A. Quick and forceful
 B. Controlled, systematic, and monitored
 C. Done by site personnel only
 D. Skipped if the load was stable
 Answer: B

- If securement shows wear or strain:
 A. Tighten and reuse
 B. Tag and remove from service immediately
 C. Leave for yard mechanics to notice
 D. Only replace if DOT cites it
 Answer: B

- Escort debriefs are conducted to:
 A. Socialize and unwind
 B. Capture operational lessons and communication performance
 C. Check fuel receipts
 D. Review customer invoices
 Answer: B

- Documentation should include:
 A. Only hours driven
 B. Mileage, hazards, escort notes, maintenance items, and compliance
 C. Nothing unless asked
 D. DOT citations only
 Answer: B

### Summary

A professional oversize operator completes the mission from start to finish.
 Post-trip procedures verify equipment condition, document performance, reinforce communication standards, and preserve a safe, ready fleet for the next movement. Thorough shutdowns build reliability and credibility.